SMARTBRANCH 360
Fault Scenario Cards

Five troubleshooting scenarios with fault, symptom, root cause, fix, and verification.

# Fault Card 1 – Wrong Default Gateway

| Fault | EMP-PC1 is configured with an incorrect default gateway. |
| --- | --- |
| Symptom | EMP-PC1 cannot reach the router or other VLANs. |
| Root Cause | The PC is using a gateway other than 192.168.10.1 for VLAN 10. |
| Fix | Set EMP-PC1 Default Gateway to 192.168.10.1 in Desktop → IP Configuration. |
| Verification | Run ping 192.168.10.1 and ping 192.168.30.10. Successful replies confirm the fix. |

# Fault Card 2 – Missing VLAN on Trunk

| Fault | VLAN 30 is missing from the trunk between switches/devices. |
| --- | --- |
| Symptom | SERVER1 in VLAN 30 cannot be reached although other connectivity works. |
| Root Cause | VLAN 30 is not allowed or active on the required trunk. |
| Fix | Run show interfaces trunk and ensure VLAN 30 is allowed and active. |
| Verification | Run ping 192.168.30.10 from EMP-PC1 and confirm successful replies. |

# Fault Card 3 – DHCP Failure

| Fault | A client fails to receive an IP address automatically. |
| --- | --- |
| Symptom | The client has no usable address/gateway and cannot communicate normally. |
| Root Cause | The DHCP pool/service or VLAN path is incorrectly configured. |
| Fix | Check the DHCP pool, network, gateway, VLAN and trunk path for the affected VLAN. |
| Verification | Renew DHCP on the client and verify an address from the correct subnet and successful ping. |

# Fault Card 4 – ACL Blocking DNS

| Fault | An ACL unintentionally blocks DNS traffic. |
| --- | --- |
| Symptom | Clients can use IP addresses but cannot resolve names. |
| Root Cause | The ACL denies DNS traffic to the DNS server or blocks DNS port 53. |
| Fix | Review the ACL and permit the required DNS traffic while preserving security rules. |
| Verification | Test DNS name resolution and verify the DNS server is reachable. |

# Fault Card 5 – NAT/Internet Failure

| Fault | Internal clients cannot access the Internet. |
| --- | --- |
| Symptom | Internal VLAN communication works but external connectivity fails. |
| Root Cause | NAT, the outside interface, or the router's default route is missing or incorrect. |
| Fix | Verify inside/outside interfaces, NAT configuration and the default route. |
| Verification | Test an external destination and verify NAT translations on the router. |

# Evidence Checklist

Capture the fault symptom before fixing it.

Record the command/configuration used to identify the cause.

Capture the corrected configuration.

Run a ping, DNS, or connectivity test after the fix.

Keep smartbranch.pkt as the working baseline.