SMARTBRANCH 360

Secure Branch Office Network using Cisco Packet Tracer

Project Documentation
Final Packet Tracer Project: smartbranch.pkt

# 1. Aim

To design and implement a branch office network in Cisco Packet Tracer with separate Employee, Guest, Server, and Management VLANs, wireless guest connectivity, inter-VLAN routing, and connectivity testing.

# 2. Objectives

Design a branch-office network using one router, two switches, a wireless access point, a server, and multiple endpoints.

Create separate VLANs for Employee, Guest, Server, and Management traffic.

Configure trunk links and Router-on-a-Stick inter-VLAN routing.

Configure wired employees, a server, a management workstation, and guest wireless clients.

Verify network connectivity using ping tests.

# 3. Software and Requirements

| Item | Requirement |
| --- | --- |
| Simulation Tool | Cisco Packet Tracer |
| Network Devices | 1 Router, 2 Switches, 1 Wireless AP |
| End Devices | 8 Employee PCs, 1 Management PC, 1 Server, 2 Guest Laptops |
| Final Project File | smartbranch.pkt |

# 4. Network Topology

The topology contains R1, SW1, SW2, SERVER1, MGMT-PC, eight employee PCs, AP1, and two guest laptops. R1 connects to SW1; SW1 connects to SW2, SERVER1, MGMT-PC, and AP1; AP1 provides wireless access to the guest laptops.

Topology structure:
R1 → SW1 → SW2 → Employee PCs
SW1 → SERVER1
SW1 → MGMT-PC
SW1 → AP1 → Guest Laptops

# 5. VLAN and IP Address Plan

| VLAN | Name | Network | Gateway | Purpose |
| --- | --- | --- | --- | --- |
| 10 | EMPLOYEE | 192.168.10.0/24 | 192.168.10.1 | Employee wired devices |
| 20 | GUEST | 192.168.20.0/24 | 192.168.20.1 | Guest wireless devices |
| 30 | SERVER | 192.168.30.0/24 | 192.168.30.1 | Internal server |
| 99 | MANAGEMENT | 192.168.99.0/24 | 192.168.99.1 | Network management |

# 6. Device IP Addressing

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| R1 Gi0/0.10 | 192.168.10.1 | 255.255.255.0 | — |
| R1 Gi0/0.20 | 192.168.20.1 | 255.255.255.0 | — |
| R1 Gi0/0.30 | 192.168.30.1 | 255.255.255.0 | — |
| R1 Gi0/0.99 | 192.168.99.1 | 255.255.255.0 | — |
| SW1 VLAN 99 | 192.168.99.2 | 255.255.255.0 | 192.168.99.1 |
| SERVER1 | 192.168.30.10 | 255.255.255.0 | 192.168.30.1 |
| MGMT-PC | 192.168.99.10 | 255.255.255.0 | 192.168.99.1 |
| GUEST-LAP1 | 192.168.20.11 | 255.255.255.0 | 192.168.20.1 |
| GUEST-LAP2 | 192.168.20.12 | 255.255.255.0 | 192.168.20.1 |
| EMP-PC1 | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| EMP-PC2 | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 |
| EMP-PC3 | 192.168.10.13 | 255.255.255.0 | 192.168.10.1 |
| EMP-PC4 | 192.168.10.14 | 255.255.255.0 | 192.168.10.1 |
| EMP-PC5 | 192.168.10.15 | 255.255.255.0 | 192.168.10.1 |
| EMP-PC6 | 192.168.10.16 | 255.255.255.0 | 192.168.10.1 |
| EMP-PC7 | 192.168.10.17 | 255.255.255.0 | 192.168.10.1 |

# 7. Configuration Procedure

## 7.1 Physical Connections

R1 Gi0/0 → SW1 Fa0/1 using Copper Straight-Through.

SW1 Fa0/2 → SW2 Fa0/1.

EMP-PC1–EMP-PC8 → SW2 Fa0/2–Fa0/9.

MGMT-PC → SW1 Fa0/10.

SERVER1 → SW1 Fa0/11.

AP1 → SW1 Fa0/12.

WPC300N wireless adapters installed in guest laptops and associated with AP1.

## 7.2 VLAN Configuration

VLAN 10 – EMPLOYEE

VLAN 20 – GUEST

VLAN 30 – SERVER

VLAN 99 – MANAGEMENT

## 7.3 Trunk Configuration

Trunking was configured on SW1 Fa0/1 toward R1, SW1 Fa0/2 toward SW2, and SW2 Fa0/1 toward SW1. VLANs 10, 20, 30, and 99 were carried across the trunk links.

## 7.4 Router-on-a-Stick

| Subinterface | VLAN | Gateway |
| --- | --- | --- |
| Gi0/0.10 | 10 | 192.168.10.1 |
| Gi0/0.20 | 20 | 192.168.20.1 |
| Gi0/0.30 | 30 | 192.168.30.1 |
| Gi0/0.99 | 99 | 192.168.99.1 |

## 7.5 Switch Port Assignment

| Switch | Port(s) | Assignment |
| --- | --- | --- |
| SW1 | Fa0/1 | Trunk to R1 |
| SW1 | Fa0/2 | Trunk to SW2 |
| SW1 | Fa0/10 | Management VLAN 99 |
| SW1 | Fa0/11 | Server VLAN 30 |
| SW1 | Fa0/12 | Guest VLAN 20 / AP1 |
| SW2 | Fa0/1 | Trunk to SW1 |
| SW2 | Fa0/2–Fa0/9 | Employee VLAN 10 |

## 7.6 Wireless Configuration

| Setting | Value |
| --- | --- |
| SSID | GUEST_WIFI |
| Authentication | WPA2-PSK |
| Encryption | AES |
| Channel | 6 |
| Password | Guest@12345 |

# 8. Testing and Verification

Connectivity was tested from EMP-PC1. The tests demonstrated connectivity to the router, server, guest network, and management host.

| Test | Destination | Result |
| --- | --- | --- |
| Employee → Router | 192.168.10.1 | Successful replies |
| Employee → Server | 192.168.30.10 | Successful replies |
| Employee → Guest | 192.168.20.11 | Successful replies |
| Employee → Management | 192.168.99.10 | Successful replies |

The first ping can time out while ARP resolution occurs; subsequent replies confirmed the paths were operational.

# 9. Screenshots

- Figure1: Final SmartBranch 360 topology.

- Figure 2: R1 show ip interface brief showing subinterfaces.

- Figure 3: SW1 show vlan brief.

- Figure 4: SW1 show interfaces trunk.

- Figure 5: SW2 VLAN 10 employee port assignment.

- Figure 6: AP1 GUEST_WIFI configuration.

- Figure 7: SERVER1 IP configuration.

- Figure 8: Guest laptop wireless association.

- Figure 9: EMP-PC1 IP configuration.

- Figure 10 : Final ping results.

# 10. Result

The SmartBranch 360 network was successfully designed and implemented in Cisco Packet Tracer. Employee, Guest, Server, and Management VLANs were created; trunking and Router-on-a-Stick routing were configured; wireless guest connectivity was established; and connectivity was verified through ping tests.

# 11. Conclusion

The project demonstrates practical VLAN segmentation, trunking, wireless access, IP addressing, and inter-VLAN routing in a branch-office environment. The working Packet Tracer project was saved as smartbranch.pkt.